# PNEUMATIC SYSTEM PRESENTATION

SYSTEM INTRODUCTION

The Airbus single aisle family pneumatic system supplies High Pressure (HP) air for:

- air conditioning,

- wing ice protection,

- water pressurization,

- hydraulic reservoir pressurization,

- engine starting.

High Pressure air can be supplied from three sources:

- the engine bleed system,

- the APU,

- a HP ground power unit.

The pneumatic system operates pneumatically and is monitored by 2 Bleed Monitoring Computers (BMC 1 & 2). There is one BMC for each engine bleed system. Both BMCs exchange data. If one BMC fails, the other BMC takes over most of its monitoring functions.

SYSTEM INTRODUCTION